www.bseltd.co.nz

Congratulations on your new surface – please read and adhere to the following;

How to use your drive/car park for the first few months after we have covered it with CRS2 Emulsion.

CRS2 Emulsion is a membrane and it takes a minimum of 28+ days to firm, to ensure the best results please follow the recommended use.

1 - Keep your vehicle speed down

2- Do not turn the steering wheel whilst the vehicle is stationary – make sure the vehicle is in motion

3- Be very careful when using Quad bikes, do not turn sharply

4- If vegetation grows through the surface spray with a weed killer (do not pull the vegetation out)

5- If the excess chip starts mounding, either sweep it off or call us to sweep the surface (we will quote for this)

6- Drive front wheel drive cars very cautiously

7- Avoid placing sharp objects i.e. motor cycle stands, and high heels, directly on the new surface

8- If there is an uphill section on your drive be careful with throttle control on the way up and braking habits on the way down the drive

9-	Do not drive near or off the edges of your driveway – may fracture the Emulsion and crack the whole edge

10- Do not Park the car or camper in the same spot all the time

11- Do not allow heavy-weight vehicles on your driveway, your driveway is constructed for automobile traffic only

12- If possible, do not use forklifts over the surface for a least 1 month, if you need to, be very careful when turning and keep the machine moving.

Reasons – Emulsion, Bitumen, or Asphalt contains certain oils or volatiles to make it more workable or malleable. Until they evaporate, the blacktop will remain soft. This aging or curing process may take as long as a year, depending on thickness, weather, etc.

If you have any concerns, please phone me on    0210256631

To learn more about CRS2 Emulsion visit   https://www.youtube.com/watch?v=PQGlS3zcaxs